Design Process

Our transport protocol is built around a lightweight TCP-style design tailored for TinyOS sensor networks. We use a fixed array of 10 socket entries (socket_store_t sockets[MAX_NUM_OF_SOCKETS]), and each socket keeps the full connection state: source/destination ports and addresses, sequence number tracking, two 128-byte circular buffers for sending and receiving, and the current position in the TCP state machine.

Connection setup follows the traditional three-way handshake. When a client calls connect(), it moves into the SYN_SENT state and transmits a SYN containing a randomly generated initial sequence number (Random.rand16()). A server in the LISTEN state will allocate a new socket when it receives a SYN, switch to SYN_RCVD, and reply with a SYN-ACK. Once the client returns an ACK, both endpoints advance to the ESTABLISHED state.

For data transmission, we implemented a stop-and-wait approach. The trySendQueued() function ensures only one segment is outstanding at any moment by verifying that lastSent == lastAck before sending the next piece of buffered data. Every data packet can include up to TCP_PACKET_MAX_PAYLOAD_SIZE bytes. Receivers immediately acknowledge any in-order data they get. A global 10-second retransmission timer checks all open connections and resends packets that have not yet been acknowledged.

Flow control is handled using the circular send and receive buffers, each tracked with head and tail indices. The advertised window is always set to SOCKET_BUFFER_SIZE, allowing the peer to know how much space is available. Connection teardown uses the standard four-step FIN process, with state transitions through FIN_WAIT_1, FIN_WAIT_2, CLOSE_WAIT, and LAST_ACK. One design issue we encountered was avoiding the transmission of a FIN while unsent data was still sitting in the queue; we solved this by adding a closeRequested[] flag that postpones sending the FIN until all buffered bytes are transmitted and acknowledged.

Internally, lastAck is stored as “ack − 1,” and we convert it into the correct TCP acknowledgment number when forming outgoing packets. A dedicated TransportTimer oversees the TIME_WAIT period, holding a closed connection for 5 seconds before freeing the socket to prevent accidental reuse.

Our overall state machine matches the standard ten-state TCP model, and the receive() function routes inbound packets according to both their flags (SYN, SYN-ACK, ACK, DATA, FIN) and the current connection state. This ensures consistent behavior during connection establishment, data exchange, and shutdown.